1. ELECTRIC CHARGE

Charge is the basic property a matter  which is responsible for electrical and magnetic effects. The excess or deficiency of electrons in a body is the causes of net charge on a body.

Types of charge :

(i)	Positive charge : It is the deficiency of electrons as compared to protons.

(ii)	Negative charge : It is the excess of electrons as compared to protons.

SI unit of charge : ampere × second i.e. coulomb;                       Dimension : [AT]

Practical units of charge are ampere × hour (=3600 C) and faraday (=96500 C)

●	1C = 3 × 109 stat coulomb, 1 absolute – coulomb = 10C, 1 Faraday = 96500C.

Specific Properties of Charge

●	Charge is a scalar quantity : It represents excess or deficiency of electrons.

● 	Charge is transferable : If a charged body is put in contact with an another body, then charge can be transferred to another body.

● 	Charge is always associated with mass

Charge cannot exist without mass though mass can exist without charge.

• 	The mass of a body changes after being charged.

•	When a body given a positive charge, its mass decreases.

•	When a body is given a negative charge, its mass increases.

● 	Charge is quantized

The quantization of electric charge is the property by virtue of which all free charges are integral multiple of a basic unit of charge represented by e. Thus charge q of a body is always given by

q = ne		n = positive or negative integer

The quantum charge is the charge that an electron or proton carries.

Note : Charge on a proton = (–) charge on an electron = 1.6 × 10–19 C

●	Charge is conserved

In an isolated system, total charge does not change with time though individual charge may change, i.e. charge can neither be created nor destroyed, Conservation of charge is also found to hold good in all types of reactions either chemical (atomic) or nuclear. No exceptions to the rule have ever been found.

●	Charge is invariant

Charge is independent of frame of reference. i.e. charge on a body does not change whatever be its speed.

Accelerated charge radiates energy

| v = 0 (i.e. at rest) Q Produces only (electric field) | v = constant  produces both and (magnetic field) but no radiation | v constant (i.e. time varying)  Q produces , and radiates energy |
| --- | --- | --- |

Numerical Problems

1.  When a piece of polythene is rubbed with wool, a charge of –2 × 10–7 C is developed on polythene. What is the amount of mass, which is transferred to the polythene ?

2.   1012 α-particles (nuclei of helium) fall per second on a neutral sphere; calculate the time in which the sphere gets charged by 2 µC.

3. In a neutral sphere, 5 × 1021 electrons are present. If 10 percent electrons are removed, then      calculate the charge on the sphere.

4. Can a body have a charge of (a) 0.32 × 10–18 C (b) 0.64 × 10–20 C (c) 4.8 × 10–21 C ?

5. A polythene piece rubbed with wool is found to have a negative charge of 3.2×10–7 C. Find the

(a) 	number of electrons transferred.

(b) 	mass gained by the polythene

2. CONDUCTOR AND INSULATORS

Conductors : Materials in which the outer electrons of each atom or molecule are weakly bound and these electrons are almost free to move throughout the body of the material are known as conductors. These materials conduct electricity easily .

All metals conduct electricity because they have some loosely bound free electrons. Silver is the best conductor.

But because of the high cost of silver, electric wires are made of copper, or in some cases aluminum.

Insulators : Materials in which all the electrons are tightly bound to their respective atoms or molecules are known as insulators. In insulators there are very few free electrons. Such materials are also called dielectrics. These materials do not conduct electricity easily .

Insulators do conduct electricity because their electrons are tightly bound to the atoms. Rubber, plastics, wood glass and porcelain are some examples of insulators. Insulators have many uses. For example, they are used as protective covers on electric wires and electrician’s tools. Certain liquids also conduct electricity. while distilled water is an insulator, addition of certain slats, acids or bases allows it to conduct electricity. Under normal circumstances, gases do not conduct electricity.

NOTE : Most nonmetallic solids do not conduct electricity. Although diamond and graphite are both forms of carbon (a nonmetal,) graphite is a conductor while diamond is an insulator.

3.Electric current

Electric charges in motion constitutes an electric current. Any medium having free electric charges is a conductor of electricity.

Positive charge flows from higher to lower potential and negative charge flows from lower to higher. Metals such as gold, silver, copper, aluminum etc. are good conductors. When charge flows in a conductor from one place to the other, then the rate of flow of charge is called electric current (I). If the moving charges are positive, the current is in the direction of motion of charge. If they are negative, the current is opposite to the direction of motion. If a charge ΔQ crosses an area in time Δt then the average electric current through the area, during this time is

Average current Iav = ΔQΔt

NOTE :  Flow of charge per second through any cross-sectional area of a conductor is called electric current. It is denoted by I or I . The S.I. unit of electric current is ampere (A) .

[ i.e., I = Q/t     OR    I = ne/t   Where n = no. of electron passes through a cross-sectional area in time t and e = charge on an electron ]

4.POTENTIAL DIFFERENCE

The potential difference between two points A and B is the work done per unit charge in taking a charge from B to a. We express this mathematically as

Here, V is the potential difference between the poets A and B, and VA and VB are the potentials at these points.

The potential at infinity is chosen as zero.

If B be the reference point, the potential at B is VB = O. From Equation, the potential at A is VA = W/q. So, the potential at a point is the work done per unit charge in taking a charge to that point from a chosen reference point. Equation may also be written as                   W = qV.

The work done on the charge q is stored as the electric potential energy (U) of the group of charges. So,                                                                                                                  U = qV

UNIT OF POTENTIAL DIFFERENCE

The unit of potential difference (and potential) is the volt, whose symbol is V. One volt is the potential difference between two points in  a current carrying conductor when 1 joule of work is done to move a charge of 1 coulomb from one point to the other.

The potential difference between two points is sometimes also called the voltage.

5.ELECTRIC CIRCUITS AND MEASURING INSTRUMENTS

A closed path in which a current can flow is called an electric circuit. An  electric circuit may have one or more electric elements such as bulbs (or lamps), cells, switches (or plug keys), metal wires, etc. Each element of a circuit has a specific function to play. For example, wires can be used to connect one element to the next. And a plug key or a switch can be used to either complete or break the closed path, thereby starting or stopping the current in the circuit .

Some common circuit elements and their symbols are shown in Figure.

5. COMMON MEASURING INSTRUMENTS

The electric current in a circuit is measured by an instrument called the ammeter, and the potential difference between two points in it is measured by a voltmeter (in voltage stabilizers). In these meters, a needle moving over a graduated scale gives the value of the measured quantity. Each meter has two terminals. The terminal marked ‘+’ is connected by a wire to the higher-potential side of a circuit, while the terminal marked ‘-‘ is connected to the lower-potential side.

USING AN AMMETER TO MEASURE CURRENT

To measure the current through an element of a circuit, an  ammeter is connected in such a way that the current flowing through it also flows through the element. Such a connection is called a series connection. In Figure, the current I flowing through the lamp also flows through the ammeter. The reading of the ammeter gives the current through the lamp. Note that if the ammeter is removed , there will be a gap, and the current through the circuit will stop .

Two or more electric elements are said to be connected in series if the current flowing through one also flows through the rest.

An ammeter is always connected in series in a circuit.

USING A VOLTMETER TO MEASURE POTENTIAL DIFFERENCE

Figure shows a circuit that two lamps connected to a cell. We want to measure the potential difference across the lamp L2, i.e., between the points A and B. As A is on the side of the positive terminal of the cell, its potential is higher than to B. So, the ‘+’ terminal of the voltmeter is connected to A, and the ‘-‘ terminal, to B. The reading of the voltmeter gives the potential difference across L2. The current flowing through the voltmeter is different from those flowing through the other elements of the circuit. Also , even if the voltmeter is removed, the current continues to flow in the circuit. Note that the potential difference across L2 and the voltmeter is the same. Such a  connection is called a parallel connection.

Two or more electric elements are said to be connected in parallel if the same potential difference exists across them.

A voltmeter is always connected in parallel in a circuit.

6. OHM’S LAW

At constant temperature the potential difference applied between ends of a conductor is directly proportional to the current flowing through.

If a potential difference V is applied to an element and a current  I or i passes through it,

Then        V α I or i

V = IR

here R is a constant for the given element (metallic wire) at a given temperature and is called its resistance.

7. RESISTANCE

It is the property of a conductor to resist the flow of electric currents or charges through it.

From equation, 				I or

So, for a given potential difference  		I or

Thus, for a given potential difference, the current is inversely proportional to the resistance. The higher is the resistance, the lower is the current. If the resistance is doubled, the current is halved. Good conductors have low resistance, while insulators have very high resistance.

UNIT OF RESISTANCE

Potential difference is measured in volts, and current is measured in amperes. From Equation, R = V/i. So, the unit of resistance is volt/ampere. This unit is called the ohm, and its symbol is . We can define one ohm as follows.

If a potential difference of 1 volt is applied across an element, and a current of 1 ampere passes through it, the resistance of element is called 1 ohm,

[ohm = Ω]

ON WHAT DOES RESISTANCE DEPEND ?

The resistance of the conductor depends (i) on its length, (ii) on its area of cross-section, and (iii) on the nature of its material. Resistance of a uniform metallic conductor is directly proportional to its length () and inversely proportional to the area of cross-section (A).

and

Combining eqs. We get

or

where(rho) is a constant of proportionality and is called electrical resistivity or specific resistance of the material of the conductor. The SI unit of resistivity is  m.

8. SERIES AND PARALLEL CONNECTIONS OF RESISTORS

A conducting material (e.g., a wire) of a particular resistance meant for use in a circuit is called a resistor. A resistor is sometimes simply referred to as a resistance. It is represented by the symbol             . Two  or more resistors can be connected in series, in parallel or in a manner that is a combination of these two.

SERIES CONNECTION OF RESISTORS

Two or more resistors are said to be connoted in series if current flowing through one also flows through the rest. The total potential difference across the combination of resistors connected in series is equal to the sum of the potential differences across the individual resistors.

V = V1 + V2 + V3

EQUIVALENT RESISTANCE IN SERIES CONCESSION

Figure (a) shows three resistors of resistances R1, R2 and  R3 connected in series. The cell connected across the combination maintains a potential difference V  across the combination. The current through the cell is i. The same current i flows through each resistor.

Let us replace the combination of resistors by a single resistor Req such that current does not change, i.e., it remains i. This resistance is called the equivalent resistance of  the combination, and its value is given by

Ohm’s law as  Req  = V / i

Thus 		V = iReq.

(a)					        (b)

The potential differences V1, V2 and V3 across the resistors R1, R2 and R3 respectively are given by

Ohm’s law as : V1 = iR1 , V2 = iR2 , V3 = iR3

Since the resistors are in series, V = V1 + V2 + V3

Substituting the values of the potential differences in the above equation,

iReq = iR1 + iR2 + iR3

or		iReq = i(R1 + R3)

or 		Req = R1 + R2 + R3

Similarly, for n resistors connected in series,

Equivalent resistance of n resistors in series : Req = R1 + R2 + R3 +……….+ Rn

PARALLEL CONNECTION OF RESISTORS

The total current flowing into the combination is equal to the sum of the currents passing through the individual resistors.

i = i1 + i2 + i3

If resistors are connected in such a way that the same potential difference gets applied to each of them, they are said to be connected in parallel.

EQUIVALENT RESISTANCE IN PARALLEL CONNECTION

Figure (a) shows three resistors of resistances R1, R2 and R3 connected in parallel across the points A and B . The cell connected across these two points maintains a potential difference V across each resistor. The current through the cell is i. It gets divided at A in to three parts i1, i2 and i3. which flow through R1, R2 and  R3 respectively.

(a)                                           (b)

Let us replaced the combination of resistors by an equivalent resistor Req such that the current i in the circuit does not change (Fig). The equivalent resistance is given by Ohm’s law as Req = V/I .

Thus,

i

The currents i1, i2 and i3 through the resistor R1, R2 and R3 respectively are given by Ohm’s law as

Since the resistors are in parallel ,

i = i1 + i2 + i3

Substituting  the values of the currents in above equation,

or

Similarly, if there are n resistors connected in parallel , their equivalent resistance Req is given by

Equivalent Resistance of n resistors in parallel :

For two resistances R1 and R2 connected in parallel.

or

The equivalent resistance in parallel connection is less than each of the resistances.

When a resistance is joined parallel to a comparatively smaller resistance, the equivalent resistance is very close to the value of the smaller resistance.

NOTE : If a resistor connected  in series with others is removed or fails, the current through each resistor becomes zero. On the other hand, if a resistor connected in parallel with others fails or is removed, the current continues to flow through the other resistors.

DISTRIBUTION OF CURRENT IN TWO RESISTORS IN PARALLEL

Consider the circuit in fig. The resistors R1 and R2 are connected in parallel. The current i gets distributed in the two resistors.

i = i1 + i2              …(i)

Applying Ohm’s law to the resistor R1

…(ii)

And applying Ohm’s law to the resistor R2

…(iii)

From (ii) and (iii),				or

Substituting for i2 in (i), we have

or

Similarly,

Thus,

The current through each branch in parallel combination of resistors is inversely proportional to its resistance

NUMERICALS

1. A metal wire of resistance R is cut into three equal pieces which are then connected side by side to form a new wire, the length of which is equal to one third of the original length.
The resistance of this new wire is :

(1) R 			(2) 3R 			(3) 			(4)

2. Three resistances of values 2, 3 and 6 are to be connected to each other an effective resistance of 4 . This can be done by connecting :

(1) 3 resistance in series with a parallel combination of 2 and 6

(2) 6resistance in series with a parallel combination of 2 and 3

(3) 2 resistance in series with a parallel combination of 3 and 6

(4) 2 resistance in parallel with a parallel combination of 3 and 6

3.What will be the equivalent resistance between the points A and D ?

(1) 10 		(2) 20  		(3) 30  		(4) 40 

4.In the circuit shown here, what is the value of the unknown resistance R so that the total resistance of the circuit between points 'P' and 'Q' is also equal to R:

(1) 30 			(2) 		(3) 		(4) 10 

5. The resistance across P and Q in the given figure is

(1) 			(2) 			(3) 2R			(4) 6R

6. The resistance of the circuit between A and B is :

(1) r 			(2) 0.5r			(3) 2r 			(4) 3r

7. Thirteen resistances each of resistance R are connected in the circuit as shown in the figure. The effective resistance between A and B is :

(1) 		(2) 2R 		(3) R  		(4)

8.The total resistance between x and y in ohms is :-

(1) 1 			(2) 4 		(3)  		(4)

9.The resultant resistance of n wires each of resistance r ohms is R, when they are connected in parallel. When these n resistances are connected in series, the resultant resistance will be :

(1) 			(2) 			(3) nR 			(4) n2R

10. For the network of resistance shown in the fig. the equivalent resistance of the network between the points A and B is 18 . The value of unknown resistance R is :

(1) 8 			(2) 10 		(3) 16 		(4) 24 

11. In the arrangement of resistances shown below, the effective resistance between points A and B is :

(1) 20 		(2) 30 		(3) 90 		(4) 110 

12. In the circuit shown the equivalent resistance between A and B is

(1) R			(2) 2R/5		(3) R/3			(4) 2R

13. In the figure the numerical values denote resistance in SI units. The total resistance of the circuit between a & b will be :

(1) 12 ohms 		(2) 24 ohms		(3) 15 ohms		(4) 6 ohms

14. Resistors R1 and R2 have an equivalent resistance of 6 ohms when connected in the circuit shown below. The resistance of R1 could be (in ) :

(1) 1			(2) 5			(3) 8			(4) 4

ANSWER

1.	(3)	2.	(3)	3.	(3)	4.	(3)	5.	(1)                     6.	(2)

7.	(4)	8.	(3)	9.	(4)	10.	(3)	11.	(1)	         12.	(1)

13.	(4)	14.	(3)